nvr_ocx explain

2018.9.13

- 1. API Description

# For more details, please see NVR/XVR(.h) or demo.

1.1）Initialization

1.1.1）NET_Startup

//------------------------------------------------------------

// Description: This API is used to initialize SDK. Please call // 					this API before calling any other API.

//------------------------------------------------------------

NET_DLLAPI BOOL  FUN_CALL NET_Startup();

1.1.2）NET_Cleanup

//------------------------------------------------------------

// Description: This API is used to release SDK resource. Please //					calling it before closing the program. //------------------------------------------------------------

NET_DLLAPI void FUN_CALL  NET_Cleanup();

1.1.3）NET_SetWaitTime

//------------------------------------------------------------

// Description: Default timeout of SDK to establish a connection  //					is 3 seconds.

// 	Input:  		WaitTime -- Timeout,unit: ms, value range:	 //								   [500,50000], the actual max timeout //									time is different with different   //									system connecting timeout.

//					TryNum -- Connecting attempt times.

//					TryInterval -- The time interval of each

//										connection.

//------------------------------------------------------------

NET_DLLAPI void FUN_CALL NET_SetWaitTime(int WaitTime=5, int TryNum=-1, int TryInterval=10);

1.1.4）NET_SetAlarm

//------------------------------------------------------------

// Description: Set whether to obtain the alarm status, if access, //				 set the time interval can not be less than 1000ms.

//------------------------------------------------------------

NET_DLLAPI void FUN_CALL NET_SetAlarm(int IsGetAlarm=1, int TryInterval=1000);

1.2）Live Preview

1.2.1）NET_ClientStart

//------------------------------------------------------------

// Description: Start live preview.

// Input:  		pChaninfo -- Live view parameter, channel is 0, //					 			 main stream or sub stream, play handle.

// 					 dataFun -- H264 data callback function.

// Return: 		 Return ID on success, -1 or null on  failure.

//------------------------------------------------------------

NET_DLLAPI HANDLE FUN_CALL NET_ClientStart(CHANNEL_CLIENTINFO *pChaninfo, AVDataCallBack dataFun = NULL);

1.2.2）NET_ClientStop

//------------------------------------------------------------

// Description: Stop live preview.

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_ClientStop(HANDLE hHandle);

1.2.3）NET_ClientSetWnd

//------------------------------------------------------------

// Description: Set the window.

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_ClientSetWnd(HANDLE hHandle, HWND hWnd);

1.2.4）NET_ClientRefreshWnd

//------------------------------------------------------------

// Description: Update the window.

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_ClientRefreshWnd(HANDLE hHandle);

1.2.5）NET_GetClientLinkState

//------------------------------------------------------------

// Description: Get link state.

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_GetClientLinkState(HANDLE hHandle);

1.2.6）NET_GetBitStreamInfo

//------------------------------------------------------------

// Description: Get bitstream information.

//		  output: pBitRate.

//					pFrameRate.

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_GetBitStreamInfo(HANDLE hHandle, ULONG *pBitRate, ULONG *pFrameRate);

1.2.7）NET_ClientSetVideoParam

//------------------------------------------------------------

// Description: Setting parameters.

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_ClientSetVideoParam(HANDLE hHandle, BYTE cbBrightness, BYTE cbContrast, BYTE cbSaturation, BYTE cbSharpness, ULONG bDefault = 0);

1.2.8）NET_ClientSetVideoParam

//------------------------------------------------------------

// Description: Get the parameters.

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_ClientGetVideoParam(HANDLE hHandle, BYTE *pcbBrightness, BYTE *pcbContrast, BYTE *pcbSaturation, BYTE *pcbSharpness);

1.3）Playback Preview

1.3.1）NET_SearchRecordFile

//------------------------------------------------------------// Description: Start searching for videos.

//------------------------------------------------------------

NET_DLLAPI int FUN_CALL NET_SearchRecordFile

(CHANNEL_CLIENTINFO *pChaninfo, RTSP_PLAYBACK_GREP_INFO *pGrepInfo, RTSP_PLAYBACK_FILE_INFO fileInfo[], int bufNum, int *nCount);

1.3.2）NET_DownloadRecordFileStart

//------------------------------------------------------------

// Description: downloading for video files

//------------------------------------------------------------

NET_DLLAPI HANDLE FUN_CALL NET_DownloadRecordFileStart(CHANNEL_CLIENTINFO *pChaninfo, RTSP_PLAYBACK_FILE_INFO *pFileInfo, const char* filename);

1.3.3）NET_DownloadRecordFileStop

//------------------------------------------------------------

// Description:  downloading for video files

//------------------------------------------------------------

NET_DLLAPI void FUN_CALL NET_DownloadRecordFileStop(HANDLE hDownloadHandle);

1.3.4）NET_PlayBackStart

//------------------------------------------------------------

// Description: PlayBack

//------------------------------------------------------------

NET_DLLAPI HANDLE FUN_CALL NET_StartPlayRecord(CHANNEL_CLIENTINFO *pChaninfo, RTSP_PLAYBACK_FILE_INFO *pFileInfo, AVDataCallBack dataFun=NULL);

1.3.5）NET_PlayBackStop

//------------------------------------------------------------

// Description:  playback

//------------------------------------------------------------

NET_DLLAPI void FUN_CALL NET_PlayBackStop(HANDLE hPlayBackHandle);

1.3.6）NET_PlayBackPause

//------------------------------------------------------------

// Description: 1: pause, 0: recovery (recovery is not limited to //pause, fast forward, frame advance can use this command to resume //normalplay)

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_PlayBackPause(HANDLE hPlayBackHandle,int lpause);

1.3.7）NET_PlayBackSpeed

//------------------------------------------------------------

// Description: Fast Forward

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_PlayBackSpeed(HANDLE hPlayBackHandle,int speed);

1.3.8） NET_PlayBackSeek

//------------------------------------------------------------// Description: Slow Forwad

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_PlayBackSeek(HANDLE hPlayBackHandle, int lseek);

1.3.9） NET_PlayBackByFrame

//------------------------------------------------------------// Description: Single frame playback, you need to call the //NET_PlayBackStop first.

//------------------------------------------------------------

NET_DLLAPI BOOL FUN_CALL NET_PlayBackByFrame(HANDLE hPlayBackHandle);

1.3.10） NET_SearchRecordStatus

//------------------------------------------------------------// Description: Get device recording status XVR

//------------------------------------------------------------

NET_DLLAPI int FUN_CALL NET_SearchRecordStatus(CHANNEL_CLIENTINFO *pChaninfo, RTSP_PLAYBACK_GREP_INFO *pGrepInfo, RTSP_RECORD_STATUS recordStatus[], int bufNum);

1.3.11） NET_SearchRecordStatus2

//------------------------------------------------------------// Description: Get device recording status NVR

//------------------------------------------------------------

NET_DLLAPI int FUN_CALL NET_SearchRecordStatus2(CHANNEL_CLIENTINFO *pChaninfo, RTSP_PLAYBACK_GREP_INFO *pGrepInfo, RTSP_RECORD_STATUS recordStatus[], int bufNum);

1.4）Other Settings

1.4.1）NET_ClientPlayAudio

//------------------------------------------------------------// Description: Audio  //------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL NET_ClientPlayAudio(HANDLE hHandle);

1.4.2）NET_ClientStopAudio

//------------------------------------------------------------// Description: Audio

//------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL NET_ClientStopAudio(HANDLE hHandle);

1.4.3）NET_ClientCapturePicture

//------------------------------------------------------------// Description: Snapshot

//------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL NET_ClientStopRecord(HANDLE hHandle);

1.4.4）NET_ClientStartRecord

//------------------------------------------------------------// Description: Start record

//------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL NET_ClientStartRecord(HANDLE hHandle, const char* filename, DWORD dwDurationSeconds=0);

1.4.5）NET_ClientStopRecord

//------------------------------------------------------------// Description: Stop record

//------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL NET_ClientStopRecord(HANDLE hHandle);

1.4.6）NET_ClientStartRecord_Another

//------------------------------------------------------------// Description: Start another video.

//------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL NET_ClientStartRecord_Another(HANDLE hHandle, const char* filename, DWORD dwDurationSeconds=0);

1.4.7）NET_ClientStopRecord_Another

//------------------------------------------------------------// Description: Stop another video

//------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL NET_ClientStopRecord_Another(HANDLE hHandle);

1.4.8）NET_ClientStartTalk

//------------------------------------------------------------// Description: Start Talk

//------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL NET_ClientStartTalk(HANDLE hHandle);

1.4.9）NET_ClientStopTalk

//------------------------------------------------------------// Description: Stop Talk

//------------------------------------------------------------NET_DLLAPI void FUN_CALL NET_ClientStopTalk(HANDLE hHandle);

1.4.10）NET_GetClientParam

//------------------------------------------------------------// Description: Get video parameters

//------------------------------------------------------------NET_DLLAPI BOOL FUN_CALL  NET_GetClientParam(HANDLE hHandle, CLIENT_PARAM* pClientParam);

1.4.11）NET_SetDrawCallback

//------------------------------------------------------------// Description: The video shows the DC callback function

//------------------------------------------------------------NET_DLLAPI void FUN_CALL  NET_SetDrawCallback(HANDLE handle, DrawCallback dawFun);

1.4.12）NET_SetMessageCallBack

//------------------------------------------------------------// Description: For each object, set the message callback function, // 				   the new version can not call this function.

//------------------------------------------------------------NET_DLLAPI void FUN_CALL NET_SetMessageCallBack(HANDLE handle, MessageCallBack msgFun);

1.4.13）NET_SetRestoreFrame

//------------------------------------------------------------// Description: The number of frames cached in the video buffer

//------------------------------------------------------------NET_DLLAPI void FUN_CALL  NET_SetRestoreFrame(HANDLE hHandle, UINT frame );

1.4.14）NET_ShowRect

//------------------------------------------------------------// Description: Specifies the area display, Area range[(0,0),

//					(imagewidth,imageheight)],When PRC = NULL to

//					restore to the original state.

//------------------------------------------------------------

NET_DLLAPI void FUN_CALL  NET_ShowRect(HANDLE hHandle, RECT *pRc);